WAP to move  ‘Micro’ from internal to external Ram  address 35H to 39H to general purpoe Ram location 60 h to 64 h

ORG 0000H
 MOV 35H, #'M'
 MOV 36H, #'I'
 MOV 37H, #'C'
 MOV 38H, #'R'
 MOV 39H, #'O'
 MOV R2, #05H       
 MOV R0, #35H     
 MOV DPTR, #0020H  
 BACK: MOV A, @R0    
       MOVX @DPTR, A  
       INC R0        
       INC DPTR      
       DJNZ R2, BACK 
 END

WAP to move data from external Ram address starting from 0025h

To 0025h to internal Ram addresss from 45 to4Ah
ORG 0000H

MOV DPTR, #0020H     ; External memory start address

MOV R0, #45H         ; Internal RAM start address

MOV R2, #06H         ; Number of bytes to transfer

BACK: MOVX A, @DPTR   ; Read from external memory

MOV @R0, A      ; Write into internal RAM

INC R0          ; Increment destination address

INC DPTR        ; Increment source address

DJNZ R2, BACK   ; Loop until all bytes transferred

END

WAP to store string “MODERN “ on ROM space 200h .transfer that string into  Ram  location  starting at 40 h .Reverse the string and store  the reversed string from  50H  Ram  location using stack only

ORG 0000H

MOV DPTR, #0020H

MOV R0, #40H

MOV R1, #06H

CLR A

L1: CLR A

MOVC A, @A+DPTR

MOV @R0, A

PUSH ACC

INC DPTR

INC R0

DJNZ R1, L1

MOV R0, #50H

MOV R1,#06H

L2: POP ACC

MOV @R0, A

INC R0

DJNZ R1, L2

ORG 0020H

DB 'MODERN'

END